Lake Cumberland District Health Department

Existing Sewerage System Inspection Procedures

Policy:

When an electric company requires a temporary or permanent electric release from the Health Department in regards to KRS 211:350 on a business or dwelling that has an existing septic system an inspection must be made by the local health department environmentalist.

Procedure:

An existing inspection shall require:

- The Existing Sewerage System and Owner’s Affidavit form (DFS-326) must be completed and signed by the owner.

- The outlet manhole of the tank and a section of the outlet pipe must be uncovered.  Inspect the tank for structural integrity, baffles, tees, etc.

- Visually inspect the lateral field area for sewage breakouts, straight pipes, or any type of system failure.

- If necessary other system components may be uncovered to determine if system is functioning properly.

- If evidence of failure is present, a charcoal dye test may be performed.

- Systems that require repairs will follow the “Repair Policy of the Lake Cumberland District Health Department.

- The environmentalist must sign and complete the DFS-326 form in detail stating their findings and any actions necessary to correct any problems that may exist.

Date Adopted: 12/2007

Date Reviewed: 3/3/09

- Date Reviewed: 3/2/10

- Date Reviewed: 3/2/11